Mobile Device Basic Security Scan

Pre-Onboarding Checklist & Tooling Recommendation

Prepared for internal BYOD / mobile device admission review

# 1. Purpose

Before mobile phones are permitted to connect to organizational resources, each device should undergo a basic, non-invasive security scan. The objective is not deep forensic analysis, but a quick check of fundamental hardening indicators — root/jailbreak status, OS patch level, installed applications, and basic device hygiene — sufficient to make an informed admit/reject decision.

# 2. Scope

This checklist covers a one-time (or periodic) admission scan and applies to both Android and iOS devices. It is intended to be:

- Lightweight — completed in minutes per device, not a full forensic audit

- Repeatable — scriptable using standard OS tooling (ADB / libimobiledevice)

- Actionable — each item maps to a clear pass/fail or risk flag

# 3. Priority Legend

Each checklist item is tagged with a priority to guide admission decisions:

Must  — Blocking issue. Device should be rejected or remediated before admission.

Should  — Significant risk. Admit with caution / conditional approval or follow-up.

Nice to have  — Good practice, informational, not a blocker on its own.

# 4. Detailed Checklist

## 1. Device & OS Identity

| Check Item | Why It Matters | Priority |
| --- | --- | --- |
| Device make, model & serial/IMEI recorded | Establishes an auditable inventory of every device admitted to the network | Must |
| OS type and exact version identified | Determines which known vulnerabilities and support status apply | Must |
| Security patch level checked (flag if >90 days old) | Old patch levels mean known, unpatched vulnerabilities | Must |
| OS version still officially supported (not EOL) | End-of-life OS versions no longer receive security fixes | Should |

## 2. Root / Jailbreak & System Integrity

| Check Item | Why It Matters | Priority |
| --- | --- | --- |
| Root (Android) or Jailbreak (iOS) detection | Rooted/jailbroken devices bypass core OS sandboxing and security controls | Must |
| Bootloader lock status (unlocked = red flag) | Unlocked bootloaders allow custom/unsigned firmware | Must |
| Custom ROM detection | Non-stock OS builds may have stripped or altered security features | Should |
| Play Integrity API / SafetyNet result (Android) | Vendor-backed attestation of device integrity | Should |
| DeviceCheck / App Attest result (iOS) | Apple's equivalent integrity attestation | Should |
| Developer options / USB debugging (ADB) status | Leaving these on in production widens the attack surface | Must |

## 3. Lock Screen & Encryption

| Check Item | Why It Matters | Priority |
| --- | --- | --- |
| Screen lock enabled (PIN / password / biometric) | First line of defense if device is lost or stolen | Must |
| Lock type is not weak (no swipe-only / no lock) | Swipe-only or disabled locks offer effectively no protection | Must |
| Auto-lock timeout within policy (e.g. ≤ 5 minutes) | Limits the window an unattended unlocked device is exposed | Should |
| Full-disk / file-based encryption enabled | Protects data at rest if the device is physically compromised | Must |
| Remote-locate / remote-wipe capability enabled | Allows containment if the device is lost or stolen | Should |

## 4. Installed Applications

| Check Item | Why It Matters | Priority |
| --- | --- | --- |
| Full installed app / APK inventory pulled | Baseline needed to detect unauthorized or risky software | Must |
| "Install from unknown sources" / sideloading status | Sideloaded apps bypass official store vetting | Must |
| Apps installed from unofficial app stores | Third-party stores have inconsistent malware screening | Should |
| High-risk permission combinations reviewed (Accessibility, Device Admin, Draw-over-apps, SMS) | These permissions are commonly abused by spyware and banking trojans | Must |
| Installed apps checked against known-malware / blocklist hashes | Flags previously identified malicious software | Should |

## 5. Network & Connectivity Hygiene

| Check Item | Why It Matters | Priority |
| --- | --- | --- |
| VPN presence and usage reviewed | Confirms encrypted transport for sensitive traffic where required | Should |
| Saved open / insecure Wi-Fi networks reviewed | Auto-joining open networks increases MITM exposure | Should |
| Bluetooth discoverability status | Discoverable Bluetooth increases pairing/attack surface | Nice to have |
| Unrecognized VPN or proxy profiles installed | Can indicate MITM tooling or unauthorized traffic interception | Must |

## 6. Certificates

| Check Item | Why It Matters | Priority |
| --- | --- | --- |
| User-installed / untrusted root CA certificates present | A common indicator of active MITM interception on the device | Must |

## 7. Management Readiness

| Check Item | Why It Matters | Priority |
| --- | --- | --- |
| Device supports Android Enterprise work profile or Apple supervised/MDM enrollment | Confirms the device can be enrolled for ongoing policy enforcement | Should |
| SIM / eSIM present and matches expected carrier | Confirms device identity and authorized carrier association | Nice to have |

## 8. Data Backup Exposure

| Check Item | Why It Matters | Priority |
| --- | --- | --- |
| Cloud backup status reviewed (what leaves the device) | Clarifies where organizational data may be replicated outside device control | Nice to have |

# 5. Recommended Open-Source Tools

The following open-source tools can perform the checks above without requiring a paid MDM or commercial MTD (Mobile Threat Defense) product:

| Tool | Purpose | Notes |
| --- | --- | --- |
| MobSF (Mobile Security Framework) | Static/dynamic analysis of APK & IPA files | Scans installed apps for risky permissions, malware indicators, and insecure configurations. Has a web UI and REST API, so it can be scripted into the onboarding flow. |
| MVT (Mobile Verification Toolkit) | Compromise / spyware indicator check | Built by Amnesty International's Security Lab. Checks Android and iOS device backups against known indicators of compromise. |
| ADB (Android Debug Bridge) | Direct Android device query | Part of the official Android platform-tools. Pulls OS version, patch level, installed package list, and system properties directly from a connected device. |
| libimobiledevice | Direct iOS device query | Open-source library/CLI (ideviceinfo, idevicebackup2) to read basic iOS device info and installed app lists over USB without jailbreaking. |
| Objection (built on Frida) | Runtime root/jailbreak verification | Dynamically probes a device at runtime to confirm root/jailbreak status rather than relying solely on self-reported flags. |
| apktool / Androguard / APKiD | Individual APK inspection | Used to decompile and inspect any single APK flagged as suspicious during the app inventory step. |
| Headwind MDM / Flyve MDM / OpenMDM | Ongoing enrollment & policy enforcement | Open-source MDM platforms. Useful beyond the one-time scan, to continuously enforce lock-screen, encryption, and patch-level policy after a device is admitted. |
| OWASP MASTG / MASVS | Reference standard (not a tool) | A vetted checklist/specification to benchmark internal scan criteria against, so nothing significant is missed. |

# 6. Suggested Workflow

- Connect device via USB (ADB for Android, libimobiledevice for iOS).

- Pull OS version, patch level, encryption status, and lock-screen state.

- Pull the installed application/package list.

- Run root/jailbreak detection (device-reported flags + Objection for runtime verification).

- Run any suspicious or unrecognized APKs through MobSF for a deeper look.

- Check for indicators of compromise using MVT if a deeper check is warranted.

- Record results in a device inventory log with a pass / conditional / fail outcome.

- For approved devices, enroll in an MDM (e.g. Headwind, Flyve, OpenMDM) to enforce policy on an ongoing basis, not just at admission.

# 7. Notes & Limitations

This checklist establishes a baseline admission gate. It does not replace continuous mobile threat defense, network-level monitoring, or a formal BYOD policy covering data ownership, acceptable use, and incident response. Root/jailbreak detection and certificate checks in particular should be treated as strong indicators rather than absolute proof — sophisticated tampering can sometimes evade detection, so periodic re-scanning is recommended rather than a single point-in-time check.